	Тип прибора
	Код серии
	Серийный номер
	Объем памяти / тип
	Защита памяти
	Часы
	Таймер
	Счетчик циклов

	DS1990A
	01H
	+
	--
	--
	--
	--
	--

	DS1991
	02H
	+
	512/NVRAM
	3x384
	--
	--
	--

	DS1992
	08H
	+
	1K/NVRAM
	--
	--
	--
	--

	DS1993
	06H
	+
	4K/NVRAM
	--
	--
	--
	--

	DS1994
	04H
	+
	4K/NVRAM
	--
	+
	+
	+

	DS1995
	0AH
	+
	16K/NVRAM
	--
	--
	--
	--

	DS1996
	0CH
	+
	64K/NVRAM
	--
	--
	--
	--

	DS1982
	09H
	+
	1K/EPROM
	--
	--
	--
	--

	DS1985
	0BH
	+
	16K/EPROM
	--
	--
	--
	--

	DS1986
	0FH
	+
	64K/EPROM
	--
	--
	--
	--

	DS1920
	10H
	+
	16/EEPROM
	ДАТЧИК ТЕМПЕРАТУРЫ


	
	
	
	
	

	Device
	Description
	Packages
	Family Code

	DS1820
	1-Wire Digital Thermometer 16 bits EEPROM
	PR-35, 16-pin SSOP
	10H

	DS18B20
	Programmable Resolution 1-Wire Digital Thermometer 16 bits EEPROM
	TO-92, 8-pin SOIC, Flip-Chip
	28H

	DS18S20
	Programmable Resolution 1-Wire Digital Thermometer 16 bits EEPROM
	TO-92, 8-pin SOIC
	10H

	DS1822
	Econo 1-Wire Digital Thermometer
	TO-92, 8-pin SOIC,TSOC
	22H

	DS2401
	Silicon Serial Number
	TO-92, SOT-223, TSOC, CSP
	01H

	DS2404
	EconoRAM Time Chip 4096 bits RAM
	DIP, SOIC, SSOP
	04H

	DS2404S-C01
	Dual-Port Memory Plus Time 4096 bits RAM
	16-pin SOIC
	84H

	DS2405
	Addressable Switch
	TO-92, SOT-223, TSOC
	  05H 



	DS2406
	Dual Addressable Switch plus Memory 1024 bits EPROM
	TO-92, TSOC, Flip-Chip
	12H

	DS2407
	Dual Addressable Switch plus Memory 1024 bits EPROM
	TO-92, TSOC, Flip-Chip
	12H

	DS2409
	MicroLAN Coupler
	TSOC
	1FH

	DS2415
	1-Wire Time Chip
	TSOC
	24H

	DS2417
	Time Chip with Interrupt
	TSOC, Flip-Chip
	27H

	DS2423
	1-Wire RAM with Counters 4096 bits RAM
	TSOC, Flip-Chip
	1DH

	DS2430A
	1-Wire EEPROM 256+64 bits EEPROM
	TO-92, TSOC, Flip-Chip
	14H

	DS2433
	1-Wire EEPROM 4096 bits EEPROM
	PR-35, SOIC, Flip-Chip
	23H

	DS2450
	1-Wire Quad A/D Converter
	SOIC8
	20H

	DS2480B
	RS232 to 1-Wire Line Driver
	SOIC
	

	DS2490
	USB to 1-Wire Bridge Chip
	24-pin SOIC, Flip-Chip
	

	DS2502
	Add-only Memory 1024 bits EPROM
	TO-92, 8-pin SOIC, TSOC, CSP
	09H

	DS2502-E64
	IEEE EUI-64 Node Address Chip
	TO-92, TSOC
	89H

	DS2502-UNW
	UniqueWare 1024 bits EPROM
	TO-92, TSOC, 8-pin SOIC
	89H

	DS2505
	Add-only Memory 16,384 bits EPROM
	TO-92, TSOC
	0BH

	DS2505-UNW
	UniqueWare 16,384 bits EPROM
	TO-92, TSOC
	8BH

	DS2506
	Add-only Memory 65,536 bits EPROM
	PR-35, SOIC
	0FH

	DS2506-UNW
	UniqueWare 65,536 bits EPROM
	PR-35, SOIC
	8FH

	DS2890
	1-Wire Digital Potentiometer
	6-pin TSOC, TO-92, Flip-Chip
	2CH

	DS9502
	ESD Protection Diode
	TSOC
	

	DS9503
	ESD Protection Diode with Resistors
	TSOC, Flip-Chip
	


